| Version | Date | Description of Revisions |
| --- | --- | --- |
| 1 | March 21, 2016 | New Spec to replace 02721 Granular Base, 02723 Granular Subbase and based on OPSS 314 (AAM) |

NOTE:

This is a CONTROLLED Document. Any documents appearing in paper form are not controlled and should be checked against the on-line file version prior to use.

Notice: This Document hardcopy must be used for reference purpose only.

The on-line copy is the current version of the document.

# GEneral

## Related Sections

### [Under "Related Sections", identify other Sections that are related to, and/or dependent on, the work results or information specified elsewhere.  The list should be limited to Sections with specific information that the reader might expect to find in this Section, but is specified elsewhere.  For example, if hardware for aluminum entrances is specified in the aluminum entrance Section, a cross-reference would be appropriate in the finish hardware Section.  The purpose of this cross-referencing is for information only, to aid in finding those other requirements—not to define the scope of the Section.

### Cross-referencing here may also be used to coordinate assemblies or systems whose components may span multiple Sections and which must meet certain performance requirements as an assembly or system.

### Contractor is responsible for coordination of the Work. Contractor is responsible for being familiar with and incorporating all required elements of cross-referenced Specifications cited.

### This Section is to be completed/updated during the design development by the Consultant. If it is not applicable to the section for the specific project it may be deleted.]

### [List Sections specifying installation of products supplied but not installed under this Section and indicate specific items.]

### Section [______ – ____________]:  Execution requirements for ...[item]...  specified under this Section.

### [List Sections specifying products installed but not supplied under this Section and indicate specific items.]

### Section [______ – ____________]:  Product requirements for ...[item]...  for installation under this Section.

### [List Sections specifying related requirements.]

### Section [______ – ____________]:  [Optional short phrase indicating relationship].

#### Section 02701 – Aggregates General

## References

### Ontario Provincial Standards for Roads and Public Works

#### OPSS 314 Untreated Granular Subbase, Base, Surface, Shoulder And Stockpiling (Nov 2013)

#### OPSS.MUNI 1010 Aggregates- Base, Subbase, Select Subgrade and Backfill Material (April 2013)

# PRODUCTS

## Materials

### Granular base: material in accordance with Section 02701 - Aggregates - General

# EXECUTION

## OPSS Amendments

### OPSS 314 shall be followed with the following amendments:

### 314.07	CONSTRUCTION

#### 314.07.01 Granular Subbase, Base, and Surface is amended by the addition of the following to the first paragraph:

##### The Contractor shall not use heavy vehicles such as tractor trailers to haul gravel if the subgrade becomes deformed. The subgrade or granular surface shall be shaped and proof rolled to ensure even and smooth surface free of dips and humps before placement of subsequent layer of material.

#### 314.07.03 Edge Ramping of Bituminous Pavement is amended by the addition of the following:

##### Where traffic must be maintained, the Contractor shall supply and maintain delineators along the shoulder at intervals of not more than 60 metres until the shoulders have been constructed.  Delineators shall be weighted in order to prevent overturning.

#### 314.07.04 Shoulders is amended by the addition of the following:

##### Normal cross fall for the new shoulder is 6% except that on the high side of super-elevated sections it shall be 2%.  Width of placement shall be 2.0m with a 0.5m rounding unless otherwise specified in the Contract Documents.

#### 314.07.06 Tolerances is amended by the addition of the following:

##### The tolerances specified in this section are working tolerances only and it is expected that the Contractor will use them as such and not keep the grade consistently high or low to replace other material.

### 314.08	QUALITY ASSURANCE

#### 314.08.01 General is amended by the addition of the following:

##### The Contractor shall supply a person to assist the Consultant’s inspector in checking the grade when requested by the Consultant.

##### Place the granular base after the sub-base surface is inspected and approved by the Consultant.

### 314.09	MEASUREMENT FOR PAYMENT

#### 314.09 Measurement for Payment is amended by the addition of the following:

##### Measurement by the cubic metre will be made by Plan Quantity (theoretical) as may be revised by Adjusted Plan Quantity and the volumes will be calculated from the end areas of the Granular ‘A’ and Granular ‘B’, Type I, sections as shown on the typical cross-sections on the Drawings.

##### No measurement or payment will be made for water used for compaction or dust control.  Payment for water shall be included in the unit price of the tender item(s) for the material to be placed or the Work to be carried out.

##### The tender quantities will be the pay quantities unless any additions or deletions are requested by the Consultant.  Any additions or deletions shall be calculated from field tape measurements agreed to by the Consultant and the Contractor.  If there is no agreement on tape measurements then cross-sections shall be used to calculate the quantity.

##### Granular material for driveways is normally calculated at 150mm of Granular ‘A’ beyond the edge of the shoulder or curb, measured horizontally to the limit of grading shown on the Drawings in accordance with OPSD 350.010 and OPSD 351.010.  Any variation from this depth will be shown on the Contract Drawings.  The Contractor shall be responsible for any granular material required to maintain access as specified under Section 01550 – Traffic Control.  As the cubic metre payment for granular materials is based on the theoretical quantity calculated as specified in the Contract Documents, the Contractor shall make its own allowances in its unit prices for any loss of Granular ‘A’ into the Granular ‘B’, Type I, and for any Granular ‘A’ required for maintenance of traffic.  The Contractor shall also make allowances in its unit prices for any losses of Granular ‘B’ during its operations.

### 314.10	BASIS OF PAYMENT

#### 314.10.01 Granular A – Item, Granular A, Stockpiled – Item, Granular A, from Stockpile – Item, Granular B Type I – Item, Granular B Type I, Stockpiled – Item, Granular B Type I, from Stockpile – Item, Granular B Type II – Item, Granular B Type II, Stockpiled – Item, Granular B Type II, from Stockpile – Item, Granular B Type III – Item, Granular B Type III, Stockpiled – Item, Granular B Type III, from Stockpile – Item, Granular M – Item, Granular M, Stockpiled – Item, Granular M, from Stockpile – Item, Granular O – Item, Granular O, Stockpiled – Item, Granular O, from Stockpile – Item, Granular S – Item, Select Subgrade Material, Compacted - Item is amended by the addition of the following:

##### The Contractor shall repair or replace any granular material lost through washouts or bladed-off the roadway and no additional payment will be made for this Work.  The applicable unit prices for Granular ‘A’ and Granular ‘B’, Type I, shall apply when additions or deletions requested by the Commissioner cause changes in the tender quantities.

##### No measurement or payment will be made for water used for compaction or dust control.  Payment for water shall be included in the Contract unit price of the tender item(s) for the material to be placed or the Work to be carried out.

##### The tender quantities will be the pay quantities unless any additions or deletions are requested by the Consultant.  Any additions or deletions shall be calculated from field tape measurements agreed to by the Consultant and the Contractor.  If there is no agreement on tape measurements then cross-sections shall be used to calculate the quantity.

##### Granular material for driveways is normally calculated at 150mm of Granular ‘A’ beyond the edge of the shoulder or curb, measured horizontally to the limit of grading shown on the plans according to the applicable 300 series OPSD.  Any variation from this depth will be shown on the Contract Drawings.  The Contractor will be responsible for any granular material required to maintain access as specified under Section 01550 – Traffic Control  As the cubic metre payment for granular materials is based on the theoretical quantity calculated as specified, the Contractor shall make its own allowances in its Contract unit prices for any loss of Granular ‘A’ into the Granular ‘B’, Type I, and for any Granular ‘A’ required for maintenance of traffic.  The Contractor shall also make allowances in its Contract unit prices for any losses of Granular ‘B’ during its operations.

## END OF SECTION

[Designer Action / Considerations: The designer should specify the following in the Contract Documents and/or drawings:

- Location, line, grade, and cross-section for granular material per typical sections]